Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(a) For an instrument rating airplane. A person who applies for an instrument-airplane rating must have logged:

(1) Except as provided in paragraph (g) of this section, 50 hours of cross-country time as pilot in command, of which 10 hours must have been in an airplane; and

(2) Forty hours of actual or simulated instrument time in the areas of operation listed in paragraph (c) of this section, of which 15 hours must have been received from an authorized instructor who holds an instrument-airplane rating, and the instrument time includes:

- (i) Three hours of instrument flight training from an authorized instructor in an airplane that is appropriate to the instrument-airplane rating within 2 calendar months before the date of the practical test; and

- (ii) Instrument flight training on cross-country flight procedures, including one cross-country flight in an airplane with an authorized instructor, that is performed under instrument flight rules, when a flight plan has been filed with an air traffic control facility, and that involves—

  - A flight of 250 nautical miles along airways or by directed routing from an air traffic control facility;

  - (B) An instrument approach at each airport; and

  - (C) Three different kinds of approaches with the use of navigation systems.

See also below:

(g) Permitted credit for a combined private pilot certificate with an instrument rating.

(h) Permitted credit for use of a full flight simulator or flight training device.

(i) Permitted credit for use of an aviation training device.

(j) Exceptions for use of a flight simulator, flight training device, aviation training device, or a combination.

Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(b) For an instrument-helicopter rating. A person who applies for an instrument-helicopter rating must have logged:

(1) Except as provided in paragraph (g) of this section, 50 hours of cross-country time as pilot in command, of which 10 hours must have been in a helicopter; and

(2) Forty hours of actual or simulated instrument time in the areas of operation listed under paragraph (c) of this section, of which 15 hours must have been with an authorized instructor who holds an instrument-helicopter rating, and the instrument time includes:

- (i) Three hours of instrument flight training from an authorized instructor in a helicopter that is appropriate to the instrument-helicopter rating within 2 calendar months before the date of the practical test; and

- (ii) Instrument flight training on cross-country flight procedures, including one cross-country flight in a helicopter with an authorized instructor that is performed under instrument flight rules and a flight plan has been filed with an air traffic control facility, and involves—

  - (A) A flight of 100 nautical miles along airways or by directed routing from an air traffic control facility;

  - (B) An instrument approach at each airport; and

  - (C) Three different kinds of approaches with the use of navigation systems.

See also below:

(g) Permitted credit for a combined private pilot certificate with an instrument rating.

(h) Permitted credit for use of a full flight simulator or flight training device.

(i) Permitted credit for use of an aviation training device.

(j) Exceptions for use of a flight simulator, flight training device, aviation training device, or a combination.

Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(c) For an instrument-powered-lift rating.  A person who applies for an instrument-powered-lift rating must have logged:

(1) Except as provided in paragraph (g) of this section, 50 hours of cross-country time as pilot in command, of which 10 hours must have been in a powered-lift; and

(2) Forty hours of actual or simulated instrument time in the areas of operation listed under paragraph (c) of this section, of which 15 hours must have been received from an authorized instructor who holds an instrument-powered-lift rating, and the instrument time includes:

- Three hours of instrument flight training from an authorized instructor in a powered-lift that is appropriate to the instrument-powered-lift rating within 2 calendar months before the date of the practical test; and

- (ii) Instrument flight training on cross-country flight procedures, including one cross-country flight in a powered-lift with an authorized instructor that is performed under instrument flight rules, when a flight plan has been filed with an air traffic control facility, that involves—

  - (A) A flight of 250 nautical miles along airways or by directed routing from an air traffic control facility;

  - (B) An instrument approach at each airport; and

  - (C) Three different kinds of approaches with the use of navigation systems.

See also below:

(g) Permitted credit for a combined private pilot certificate with an instrument rating.

(h) Permitted credit for use of a full flight simulator or flight training device.

(i) Permitted credit for use of an aviation training device.

(j) Exceptions for use of a flight simulator, flight training device, aviation training device, or a combination.

Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(g) Permitted credit for a combined private pilot certificate with an instrument rating. An applicant for a combined private pilot certificate with an instrument rating may satisfy the cross-country time requirements of this section by crediting:

(1) For an instrument-airplane rating or an instrument-powered-lift rating, up to 45 hours of cross-country time performing the duties of pilot in command with an authorized instructor; or

(2) For an instrument-helicopter rating, up to 47 hours of cross-country time performing the duties of pilot in command with an authorized instructor.

Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(h) Permitted credit for use of a full flight simulator or flight training device. If the instrument time was provided by an authorized instructor in a full flight simulator or flight training device—

(1) A maximum of 30 hours may be performed in that full flight simulator or flight training device if the instrument time was completed in accordance with part 142 of this chapter; or

(2) A maximum of 20 hours may be performed in that full flight simulator or flight training device if the instrument time was not completed in accordance with part 142 of this chapter.

Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(i) Permitted credit for use of an aviation training device. A maximum of 10 hours of instrument time received in a basic aviation training device or a maximum of 20 hours of instrument time received in an advanced aviation training device may be credited for the instrument time requirements of this section if—

(1) The device is approved and authorized by the FAA;

(2) An authorized instructor provides the instrument time in the device; and

(3) The FAA approved the instrument training and instrument tasks performed in the device.

Student Name: _______________________________

§ 61.65 Aeronautical experience. (eff. Jan 21, 2025)

(j) Exceptions for use of a flight simulator, flight training device, aviation training device, or a combination. Except as provided in paragraph (h)(1) of this section, a person may not credit more than 20 total hours of instrument time in a full flight simulator, flight training device, aviation training device, or a combination towards the instrument time requirements of this section.